Materials:

Two new Boots

Two cotter pins 2mm dia by 24 mm long

Tools: 
[image: image1.jpg]



Jack stand (not shown) 
17 mm socket and ratchet handle to remove nut

3/16” wide flat screwdriver (to pry old boot)
Torque wrench (recommended)

Tie rod end puller or equivalent (I used ordinary puller with 3/8-24 forcing screw) to separate tapered stud it from the steering knuckle 

Small hammer

Tool to install new boot on the tie rod end (must have 32.5 to 32.8 mm ID and be minimum 46 mm long; I machined it from gray PVC pipe 1 and 1/4 ID). [Yellow box]
C-clamp or seal driver to force new boot on the tie rod end; I used fancy screw-type mini press I made for different application

Puller guide (if two jaw puller is used) indicated by cyan arrow
Procedure:

1. Support the car and remove wheel

2. Remove cotter pin from the stud

3. Clean the threads using brush

4. Break loose and remove 17 mm nut; reinstall the nut flush with the stud end 

5. Install 17 mm socket on the nut

6. Insert puller guide to the 3/8 square drive hole on the socket (a guide can be made from 3/8-16 socket head screw cut to 7/16 inch length)

7. Place forcing screw tip into the puller guide (cyan arrow) and install jaws on the steering knuckle:

[image: image2.jpg]


 
8. Tighten the forcing screw to pop the cone stud from steering knuckle (both studs popped out loose with the sound of the train wreck without much effort)

9. Using screwdriver and small hammer dislodge the old boot; one light tap is typically enough.

10. Now remove the nut, take the tie rod end out of the hole and take old boot off :
      [image: image3.jpg]



11. Scoop the grease from old boot and transfer it to new boot; keep 3/16” near the top free from grease
[image: image4.jpg]



12. Stick new boot into installation tool; insert stud into new boot and press the boot on the tie rod end (I used the press but C-clamp can be used as well):
[image: image5.jpg]



13. Insert stud into the steering knuckle; reinstall the nut and tighten to 32 foot pounds  

14. Install the cotter pin 
15. Put wheel back (torque to 80 foot pounds) and do the other side. 

16. Road test the car; alignment is usually NOT required after this repair 
Before: 

[image: image6.jpg]



After: 

[image: image7.jpg]


 

PAGE  
2

